Neptune MACH 10 R900 → MQTT → Home Assistant

Raspberry Pi installation and configuration guide

Purpose. Read a Neptune MACH 10 water meter non-invasively over its Neptune R900 radio, decode it locally with rtl_433, publish the reading to MQTT, and expose cumulative consumption plus leak/backflow status in Home Assistant. No plumbing modification is required.

Validated installation: Neptune MACH 10, 1½-inch meter, R900 broadcasts around 915 MHz, rtl_433 protocol 228. The installation below was proven with meter RF ID 1584089202. Replace that ID with your own meter ID if using this guide elsewhere.

# 1. Architecture

Neptune MACH 10 / R900
        ↓  902–928 MHz RF
RTL-SDR receiver (Nooelec NESDR SMArt v5)
        ↓ USB
Raspberry Pi
  rtl_433 → MQTT → filtered home topic
        ↓ LAN
Home Assistant

# 2. Hardware

| Item | Working choice | Notes |
| --- | --- | --- |
| Raspberry Pi | Pi 1 Model B+ worked | A Pi 3/4/Zero 2 W is preferable for easier builds and better performance. Ethernet is recommended. |
| SDR | Nooelec NESDR SMArt v5 | RTL2832U/R820T2 based. The included antenna worked for initial reception. |
| Storage | microSD card | Raspberry Pi OS Lite; 8 GB or larger is plenty. |
| Network | Ethernet | The Pi 1 B+ has no built-in Wi-Fi. |

# 3. Install Raspberry Pi OS

1. Flash Raspberry Pi OS Lite

For a very old Pi 1 B+, use Raspberry Pi OS (Legacy) Lite, 32-bit, Bookworm.

For newer hardware, use the current Raspberry Pi OS Lite appropriate for that model.

In Raspberry Pi Imager, set a hostname (for example watermeter), create a user, enable SSH, and configure networking as needed.

2. Boot and connect

On a Pi 1 B+, connect Ethernet before powering up.

ssh <user>@watermeter.local

# 4. Install prerequisites

sudo apt update
sudo apt full-upgrade -y
sudo apt install -y git cmake build-essential libusb-1.0-0-dev librtlsdr-dev rtl-sdr mosquitto mosquitto-clients tmux

# 5. Build and install rtl_433

On current Raspberry Pi OS, a packaged rtl_433 may be available. Check first:

apt-cache policy rtl-433

If a suitable package is available, prefer it. Otherwise build from source:

cd ~
git clone https://github.com/merbanan/rtl_433.git
cd rtl_433
mkdir -p build
cd build
cmake ..
make -j1
sudo make install
sudo ldconfig

## Old Pi build workaround

The Pi 1 B+ may become unresponsive during compilation because it has only 512 MB RAM. Use a temporary swap file and tmux.

sudo fallocate -l 1G /swapfile
sudo chmod 600 /swapfile
sudo mkswap /swapfile
sudo swapon /swapfile
free -h

tmux new -s build
cd ~/rtl_433/build
make -j1

If SSH disconnects, reconnect and resume the existing session:

tmux attach -t build

After the build succeeds, remove the temporary swap if desired:

sudo swapoff /swapfile
sudo rm /swapfile

# 6. Verify the SDR

lsusb
rtl_test -t
rtl_433 -V

A working NESDR SMArt v5 appears as a Realtek RTL2838/RTL2832U device. rtl_test should detect the R820T/R820T2 tuner.

# 7. Receive Neptune R900 packets

The working receive command was:

rtl_433 -f 915M -s 1024k -R 228 -M level

For JSON output:

rtl_433 -f 915M -s 1024k -R 228 -F json

Successful packets look like:

{"time":"2026-09-11T14:54:19","model":"Neptune-R900","id":1584089202,"unkn1":10,"metertype":3,"unkn2":4,"nouse":0,"backflow":0,"consumption":15616,"leak":2,"leaknow":1,"extra":"8c50f7"}

## Identify your meter

You may receive neighboring R900 meters too.

Run water and watch which meter ID changes.

For the validated installation, the home meter ID is 1584089202.

Confirm identity against the physical meter before automating.

## Consumption scaling for this MACH 10

The physical LCD and controlled draws established the following mapping for this meter:

1 raw consumption count = 0.1 ft³
1 raw consumption count = 0.748052 US gal

cubic feet = consumption / 10
gallons    = consumption × 0.748052

Example: raw consumption 15616 = 1561.6 ft³ = approximately 11,683.1 gal.

# 8. Configure Mosquitto on the Pi

sudo systemctl enable --now mosquitto
sudo mosquitto_passwd -c /etc/mosquitto/passwd watermeter

Create the broker configuration:

sudo nano /etc/mosquitto/conf.d/watermeter.conf

listener 1883
allow_anonymous false
password_file /etc/mosquitto/passwd

Set permissions so the Mosquitto service can read the password file:

sudo chown root:mosquitto /etc/mosquitto/passwd
sudo chmod 640 /etc/mosquitto/passwd
sudo systemctl restart mosquitto
sudo systemctl status mosquitto --no-pager

Test from another machine:

mosquitto_sub -h <PI_IP> -u watermeter -P '<MQTT_PASSWORD>' -t '#' -v

# 9. Configure rtl_433 to publish MQTT

sudo mkdir -p /etc/rtl_433
sudo nano /etc/rtl_433/watermeter.conf

frequency 915M
sample_rate 1024k
protocol 228

output mqtt://127.0.0.1:1883,user=watermeter,pass=<MQTT_PASSWORD>,retain=0,events=watermeter/events

report_meta time:iso

Test it interactively:

rtl_433 -c /etc/rtl_433/watermeter.conf

# 10. Filter to only your meter

rtl_433 will also publish neighboring R900 meters. This lightweight filter republishes only the target meter to a retained topic called watermeter/home.

sudo nano /usr/local/bin/watermeter-filter.sh

#!/bin/bash

mosquitto_sub \
  -h 127.0.0.1 \
  -u watermeter \
  -P '<MQTT_PASSWORD>' \
  -t 'watermeter/events' |
while read -r line
do
  id=$(printf '%s\n' "$line" | sed -n 's/.*"id":\([0-9]*\).*/\1/p')

  if [ "$id" = "1584089202" ]; then
    mosquitto_pub \
      -h 127.0.0.1 \
      -u watermeter \
      -P '<MQTT_PASSWORD>' \
      -t 'watermeter/home' \
      -r \
      -m "$line"
  fi
done

sudo chmod +x /usr/local/bin/watermeter-filter.sh

Replace 1584089202 with the ID of the meter being monitored on another installation.

# 11. Run everything headless with systemd

## rtl_433 service

sudo nano /etc/systemd/system/watermeter.service

[Unit]
Description=Neptune R900 Water Meter Receiver
After=network-online.target mosquitto.service
Wants=network-online.target
Requires=mosquitto.service

[Service]
Type=simple
User=watermarker
ExecStart=/usr/local/bin/rtl_433 -c /etc/rtl_433/watermeter.conf
Restart=always
RestartSec=10

[Install]
WantedBy=multi-user.target

If rtl_433 is installed elsewhere, use which rtl_433 and adjust ExecStart. If your Linux username is not watermarker, change User= accordingly.

## Filter service

sudo nano /etc/systemd/system/watermeter-filter.service

[Unit]
Description=Filter Neptune R900 Water Meter MQTT
After=mosquitto.service watermeter.service
Requires=mosquitto.service

[Service]
Type=simple
User=watermarker
ExecStart=/usr/local/bin/watermeter-filter.sh
Restart=always
RestartSec=5

[Install]
WantedBy=multi-user.target

sudo systemctl daemon-reload
sudo systemctl enable --now watermeter
sudo systemctl enable --now watermeter-filter
systemctl status watermeter --no-pager
systemctl status watermeter-filter --no-pager

Useful live logs:

journalctl -u watermeter -f
journalctl -u watermeter-filter -f

# 12. Connect Home Assistant to MQTT

1. Add the MQTT integration

In Home Assistant, go to Settings → Devices & services → Add Integration → MQTT.

Broker: Raspberry Pi IP address

Port: 1883

Username: watermeter

Password: the Mosquitto password created above

# 13. Create Home Assistant MQTT entities

Edit Home Assistant configuration.yaml. The path may appear as /config/configuration.yaml or /homeassistant/configuration.yaml depending on the installation and editor. If an mqtt: section already exists, merge these entries into it instead of creating a second mqtt: key.

mqtt:
  sensor:
    - name: "Water Consumption"
      unique_id: neptune_water_consumption
      state_topic: "watermeter/home"
      device_class: water
      state_class: total_increasing
      unit_of_measurement: "gal"
      suggested_display_precision: 1
      value_template: >
        {{ (value_json.consumption | float * 0.748052) | round(3) }}
      device:
        identifiers:
          - neptune_mach10_water_meter
        name: "Neptune MACH 10 Water Meter"
        manufacturer: "Neptune"
        model: "MACH 10"

    - name: "Water Consumption Cubic Feet"
      unique_id: neptune_water_consumption_cuft
      state_topic: "watermeter/home"
      device_class: water
      state_class: total_increasing
      unit_of_measurement: "ft³"
      suggested_display_precision: 1
      value_template: >
        {{ (value_json.consumption | float / 10) | round(1) }}
      device:
        identifiers:
          - neptune_mach10_water_meter
        name: "Neptune MACH 10 Water Meter"
        manufacturer: "Neptune"
        model: "MACH 10"

    - name: "Water Meter Raw"
      unique_id: neptune_water_meter_raw
      state_topic: "watermeter/home"
      value_template: "{{ value_json.consumption }}"
      device:
        identifiers:
          - neptune_mach10_water_meter
        name: "Neptune MACH 10 Water Meter"
        manufacturer: "Neptune"
        model: "MACH 10"

  binary_sensor:
    - name: "Water Meter Current Leak"
      unique_id: neptune_water_current_leak
      state_topic: "watermeter/home"
      device_class: problem
      value_template: >
        {{ 'ON' if value_json.leaknow | int > 0 else 'OFF' }}
      device:
        identifiers:
          - neptune_mach10_water_meter
        name: "Neptune MACH 10 Water Meter"
        manufacturer: "Neptune"
        model: "MACH 10"

    - name: "Water Meter Backflow"
      unique_id: neptune_water_backflow
      state_topic: "watermeter/home"
      device_class: problem
      value_template: >
        {{ 'ON' if value_json.backflow | int > 0 else 'OFF' }}
      device:
        identifiers:
          - neptune_mach10_water_meter
        name: "Neptune MACH 10 Water Meter"
        manufacturer: "Neptune"
        model: "MACH 10"

Check the Home Assistant configuration, restart Home Assistant, and verify that all five entities appear under one device named Neptune MACH 10 Water Meter.

# 14. Add water to the Energy dashboard

Settings → Dashboards → Energy → Water consumption → Add water source.

Choose Water Consumption.

Leave Water flow rate blank unless you later create a suitable derived flow-rate sensor.

Cost tracking is optional.

The cumulative gallons sensor is appropriate because it is configured with device_class: water and state_class: total_increasing.

# 15. Daily, weekly, and monthly usage

Create Utility Meter helpers in Home Assistant: Settings → Devices & services → Helpers → Create helper → Utility meter.

Daily Water Usage: source = Water Consumption; reset cycle = Daily.

Weekly Water Usage: source = Water Consumption; reset cycle = Weekly.

Monthly Water Usage: source = Water Consumption; reset cycle = Monthly.

# 16. Troubleshooting

| Symptom | What to check |
| --- | --- |
| Pi disappears during build | Use 1 GB temporary swap, make -j1, and compile inside tmux. On a Pi 1, unplug the SDR while compiling if power is marginal. |
| Mosquitto restart fails with status 13 | Check /etc/mosquitto/passwd permissions. Use owner root:mosquitto and mode 640. |
| Connection refused on port 1883 | Mosquitto is not listening, not running, or a firewall is blocking it. Check systemctl status mosquitto and ss -ltnp ¦ grep 1883. |
| rtl_433 sees the SDR but no meter | Use -f 915M -s 1024k -R 228. Wait several minutes. R900 packets hop within the 902–928 MHz band. |
| R82XX PLL not locked at startup | A transient warning can occur during tuning. If valid R900 packets are decoding, it is not blocking operation. |
| Multiple meter IDs appear | Expected in neighborhoods with Neptune R900 meters. Filter by the positively identified meter ID. |
| Leak sensor says Problem | The R900 leaknow flag is Neptune meter-side leak/continuous-use logic. Recent intentional water use may set it. Verify whether it clears after a period with no water use. |
| No Home Assistant statistics immediately | New total_increasing sensors need time for Recorder/long-term statistics to accumulate. |

# 17. Security and maintenance

Use a dedicated MQTT account rather than a normal personal account.

Do not leave allow_anonymous true after testing.

Avoid putting the real MQTT password into shared documentation, screenshots, or source control.

If a password is exposed, rotate it with mosquitto_passwd and update rtl_433/filter/Home Assistant credentials.

Back up /etc/rtl_433, /etc/mosquitto, /usr/local/bin/watermeter-filter.sh, and the two systemd unit files.

# 18. Upgrading the Raspberry Pi later

The design is hardware-independent. To move to a newer Pi:

Install Raspberry Pi OS Lite and the same packages.

Move the SDR to the new Pi.

Copy the rtl_433 configuration, Mosquitto configuration/password file, filter script, and systemd service files.

Update the Pi IP address in the Home Assistant MQTT integration if the address changes.

Restart the services and verify watermeter/home with mosquitto_sub.

A Pi 3/4 or Zero 2 W should make installation and compilation substantially easier while consuming very little power.

# 19. Final verification checklist

☐ rtl_433 -f 915M -s 1024k -R 228 decodes Neptune-R900 packets.

☐ The target meter ID is positively identified by controlled water use.

☐ Mosquitto requires authentication and listens on port 1883.

☐ watermeter/events contains raw R900 events.

☐ watermeter/home contains only the target meter and is retained.

☐ watermeter.service and watermeter-filter.service are enabled and active.

☐ Home Assistant MQTT integration connects to the Pi broker.

☐ Water Consumption shows the expected cumulative gallons.

☐ All entities are grouped under Neptune MACH 10 Water Meter.

☐ Water Consumption is configured as the Energy dashboard water source.