Neptune MACH 10 R900 Water Meter

Raspberry Pi 5 → MQTT → Home Assistant

Clean-install guide for a headless, wired, PoE-powered receiver

Validated project configuration Neptune MACH 10 ultrasonic meter with R900 radio, Nooelec NESDR SMArt v5, rtl_433 protocol 228, MQTT, and Home Assistant. The project meter RF ID is 1584089202. If this guide is reused elsewhere, identify and substitute the correct meter ID before enabling the filter.

# System architecture

Neptune MACH 10 / R900
        ↓  902–928 MHz RF
Nooelec NESDR SMArt v5
        ↓  USB
Raspberry Pi 5
  rtl_433 → Mosquitto → meter-ID filter
        ↓  retained MQTT topic: watermeter/home
Wired Ethernet
        ↓
Home Assistant

# 1. Hardware

| Component | Recommended | Notes |
| --- | --- | --- |
| Computer | Raspberry Pi 5 | 2 GB or more. This workload is extremely light; RAM capacity is not important. |
| Storage | 16–32 GB high-endurance microSD | More capacity is fine. High-endurance media is preferable for an unattended appliance. |
| SDR | Nooelec NESDR SMArt v5 | Known working RTL2832U/R820T2 receiver. |
| Network | Wired Gigabit Ethernet | Recommended for reliability and simple MQTT access. |
| Power | USB-C 5 V / 5 A | Full Pi 5 power target, especially with the USB SDR attached. |
| PoE option | 802.3at/PoE+ splitter with USB-C 5 V / 5 A output | Splitter should also provide Gigabit Ethernet data output if one cable is used for both power and network. |
| Cooling | Pi 5 case with fan or Active Cooler | Recommended for a permanently powered Pi 5, even though this workload is light. |

PoE wiring PoE switch → PoE+ splitter → RJ45 data to the Pi 5 Ethernet port + USB-C power to the Pi 5 power input. Avoid common 5 V / 2.5 A or 3 A splitters for a new Pi 5 installation. A lower-current supply can boot a Pi 5, but the full 5 A supply gives the best USB/peripheral power margin.

# 2. Install Raspberry Pi OS

1. Use Raspberry Pi Imager and select the current Raspberry Pi OS Lite 64-bit image for Raspberry Pi 5.

2. In Imager customization, set a hostname such as watermeter, create the administrator user, enable SSH, and configure the locale/time zone.

3. For a wired installation, connect Ethernet before first boot. If using a PoE splitter, connect both its RJ45 data lead and USB-C power lead to the Pi.

4. Boot the Pi and connect over SSH.

ssh <PI_USER>@watermeter.local

Recommended network setup Create a DHCP reservation for the Pi in your router so the MQTT broker keeps a stable IP address. Home Assistant will point at this address.

# 3. Update the OS and install prerequisites

sudo apt update
sudo apt full-upgrade -y
sudo apt install -y git cmake build-essential libusb-1.0-0-dev   librtlsdr-dev rtl-sdr mosquitto mosquitto-clients jq

# 4. Install rtl_433

Building from source ensures the Neptune R900 decoder used by this installation is available.

cd ~
git clone https://github.com/merbanan/rtl_433.git
cd rtl_433
mkdir -p build
cd build
cmake ..
make -j4
sudo make install
sudo ldconfig
rtl_433 -V

Pi 5 difference No temporary swap file or single-threaded build is normally needed on a Pi 5. The old Pi 1 workaround is intentionally omitted from this guide.

# 5. Verify the SDR

lsusb
rtl_test -t

A working NESDR should appear as a Realtek RTL2838/RTL2832U device. rtl_test should detect the R820T/R820T2 tuner. Press Ctrl+C after the tuner is identified.

# 6. Receive Neptune R900 packets

Start with the known-good receive settings:

rtl_433 -f 915M -s 1024k -R 228 -M level

For machine-readable output:

rtl_433 -f 915M -s 1024k -R 228 -F json

A packet from the project meter looks like:

{"model":"Neptune-R900","id":1584089202,"backflow":0,"consumption":15616,"leak":2,"leaknow":1}

## Identify the correct meter

You may receive several nearby Neptune R900 meters.

Run water in the house and watch which meter ID advances.

Confirm the RF reading against the physical meter display before automating.

For this project, the confirmed RF meter ID is 1584089202.

## Consumption scaling

For this MACH 10/R900 combination, controlled draws and the physical LCD established:

1 raw consumption count = 0.1 ft³
1 raw consumption count = 0.748052 US gal

cubic feet = consumption / 10
gallons    = consumption × 0.748052

Example: raw consumption 15616 = 1561.6 ft³ ≈ 11,683.1 gal.

Do not assume this scaling for every meter If you reuse this guide with a different Neptune meter or encoder configuration, verify the raw-count scaling against the physical register before using it for billing-quality totals.

# 7. Configure Mosquitto

sudo systemctl enable --now mosquitto
sudo mosquitto_passwd -c /etc/mosquitto/passwd watermeter

Create the broker configuration:

sudo nano /etc/mosquitto/conf.d/watermeter.conf

listener 1883
allow_anonymous false
password_file /etc/mosquitto/passwd

sudo chown root:mosquitto /etc/mosquitto/passwd
sudo chmod 640 /etc/mosquitto/passwd
sudo systemctl restart mosquitto
sudo systemctl status mosquitto --no-pager

Optional remote test from another computer:

mosquitto_sub -h <PI_IP> -u watermeter -P '<MQTT_PASSWORD>' -t '#' -v

# 8. Configure rtl_433 → MQTT

sudo mkdir -p /etc/rtl_433
sudo nano /etc/rtl_433/watermeter.conf

frequency 915M
sample_rate 1024k
protocol 228

output mqtt://127.0.0.1:1883,user=watermeter,pass=<MQTT_PASSWORD>,retain=0,events=watermeter/events

report_meta time:iso

Test interactively:

rtl_433 -c /etc/rtl_433/watermeter.conf

In another SSH session, verify MQTT events:

mosquitto_sub -h 127.0.0.1 -u watermeter -P '<MQTT_PASSWORD>' -t 'watermeter/events' -v

# 9. Filter to the house meter

The SDR can hear neighboring meters. The filter republishes only the selected meter to a retained topic, watermeter/home.

sudo nano /usr/local/bin/watermeter-filter.sh

#!/bin/bash

mosquitto_sub \
  -h 127.0.0.1 \
  -u watermeter \
  -P '<MQTT_PASSWORD>' \
  -t 'watermeter/events' |
while read -r line
do
  id=$(printf '%s\n' "$line" | jq -r '.id // empty')

  if [ "$id" = "1584089202" ]; then
    mosquitto_pub \
      -h 127.0.0.1 \
      -u watermeter \
      -P '<MQTT_PASSWORD>' \
      -t 'watermeter/home' \
      -r \
      -m "$line"
  fi
done

sudo chmod +x /usr/local/bin/watermeter-filter.sh

Meter ID 1584089202 is the confirmed project meter. Substitute a different verified ID only if this guide is used on another installation.

# 10. Run the receiver and filter with systemd

## rtl_433 service

sudo nano /etc/systemd/system/watermeter.service

[Unit]
Description=Neptune R900 Water Meter Receiver
After=network-online.target mosquitto.service
Wants=network-online.target
Requires=mosquitto.service

[Service]
Type=simple
User=<PI_USER>
ExecStart=/usr/local/bin/rtl_433 -c /etc/rtl_433/watermeter.conf
Restart=always
RestartSec=10

[Install]
WantedBy=multi-user.target

## Filter service

sudo nano /etc/systemd/system/watermeter-filter.service

[Unit]
Description=Filter Neptune R900 Water Meter MQTT
After=mosquitto.service watermeter.service
Requires=mosquitto.service

[Service]
Type=simple
User=<PI_USER>
ExecStart=/usr/local/bin/watermeter-filter.sh
Restart=always
RestartSec=5

[Install]
WantedBy=multi-user.target

sudo systemctl daemon-reload
sudo systemctl enable --now watermeter
sudo systemctl enable --now watermeter-filter
systemctl status watermeter --no-pager
systemctl status watermeter-filter --no-pager

Useful logs:

journalctl -u watermeter -f
journalctl -u watermeter-filter -f

# 11. Verify the full Pi-side data path

1. Confirm raw SDR decoding with rtl_433 if needed.

2. Confirm watermeter/events contains R900 packets.

3. Confirm watermeter/home contains only the target meter and is retained.

4. Confirm the services survive a reboot.

mosquitto_sub -h 127.0.0.1 -u watermeter -P '<MQTT_PASSWORD>' -t 'watermeter/events' -v
mosquitto_sub -h 127.0.0.1 -u watermeter -P '<MQTT_PASSWORD>' -t 'watermeter/home' -v
sudo reboot

After reboot:

systemctl status watermeter --no-pager
systemctl status watermeter-filter --no-pager
mosquitto_sub -h 127.0.0.1 -u watermeter -P '<MQTT_PASSWORD>' -t 'watermeter/home' -v

# 12. Connect Home Assistant to MQTT

1. In Home Assistant, open Settings → Devices & services → Add Integration → MQTT.

2. Broker: the Raspberry Pi 5 IP address. Port: 1883.

3. Username: watermeter. Password: the Mosquitto password created above.

4. Confirm the MQTT integration reports connected.

# 13. Create Home Assistant entities

Add the following to configuration.yaml. If an mqtt: section already exists, merge these entries into it instead of creating a second mqtt: key.

mqtt:
  sensor:
    - name: "Water Consumption"
      unique_id: neptune_water_consumption
      state_topic: "watermeter/home"
      device_class: water
      state_class: total_increasing
      unit_of_measurement: "gal"
      suggested_display_precision: 1
      value_template: >
        {{ (value_json.consumption | float * 0.748052) | round(3) }}
      device:
        identifiers:
          - neptune_mach10_water_meter
        name: "Neptune MACH 10 Water Meter"
        manufacturer: "Neptune"
        model: "MACH 10"

    - name: "Water Consumption Cubic Feet"
      unique_id: neptune_water_consumption_cuft
      state_topic: "watermeter/home"
      device_class: water
      state_class: total_increasing
      unit_of_measurement: "ft³"
      suggested_display_precision: 1
      value_template: >
        {{ (value_json.consumption | float / 10) | round(1) }}
      device:
        identifiers:
          - neptune_mach10_water_meter
        name: "Neptune MACH 10 Water Meter"
        manufacturer: "Neptune"
        model: "MACH 10"

    - name: "Water Meter Raw"
      unique_id: neptune_water_meter_raw
      state_topic: "watermeter/home"
      value_template: "{{ value_json.consumption }}"
      device:
        identifiers:
          - neptune_mach10_water_meter
        name: "Neptune MACH 10 Water Meter"
        manufacturer: "Neptune"
        model: "MACH 10"

  binary_sensor:
    - name: "Water Meter Current Leak"
      unique_id: neptune_water_current_leak
      state_topic: "watermeter/home"
      device_class: problem
      value_template: >
        {{ 'ON' if value_json.leaknow | int > 0 else 'OFF' }}
      device:
        identifiers:
          - neptune_mach10_water_meter
        name: "Neptune MACH 10 Water Meter"
        manufacturer: "Neptune"
        model: "MACH 10"

    - name: "Water Meter Backflow"
      unique_id: neptune_water_backflow
      state_topic: "watermeter/home"
      device_class: problem
      value_template: >
        {{ 'ON' if value_json.backflow | int > 0 else 'OFF' }}
      device:
        identifiers:
          - neptune_mach10_water_meter
        name: "Neptune MACH 10 Water Meter"
        manufacturer: "Neptune"
        model: "MACH 10"

1. Run Home Assistant configuration validation.

2. Restart Home Assistant.

3. Open the Neptune MACH 10 Water Meter device and verify all five entities have current values.

# 14. Add water to the Energy dashboard

1. Open Settings → Dashboards → Energy → Water consumption.

2. Add Water Consumption as the water source.

3. Leave Water flow rate blank. The R900 total is not an instantaneous flow sensor.

4. Configure cost tracking only if desired.

# 15. Daily, weekly, and monthly usage

Create three Utility Meter helpers under Settings → Devices & services → Helpers → Create helper → Utility meter:

| Helper | Source | Reset cycle |
| --- | --- | --- |
| Daily Water Usage | Water Consumption | Daily |
| Weekly Water Usage | Water Consumption | Weekly |
| Monthly Water Usage | Water Consumption | Monthly |

# 16. Pi 5 reliability checks

Because this system is intended to run unattended, perform these checks after installation:

vcgencmd get_throttled
lsusb
systemctl is-enabled watermeter watermeter-filter mosquitto
systemctl is-active watermeter watermeter-filter mosquitto

Ideal power-status output is:

throttled=0x0

If get_throttled reports power events, inspect the PoE splitter, PoE switch budget, USB-C cable, and connectors.

Keep the SDR plugged directly into the Pi unless a powered hub is needed for a specific installation.

Use a case/fan arrangement that does not block the SDR or antenna placement.

# 17. Troubleshooting

| Symptom | Check | Likely layer |
| --- | --- | --- |
| No data in Home Assistant | Subscribe remotely to watermeter/home | Home Assistant/MQTT if topic is updating |
| watermeter/home stale | Check watermeter-filter service and watermeter/events | Filter |
| No watermeter/events | Check watermeter service and run rtl_433 interactively | rtl_433/MQTT |
| rtl_433 sees no SDR | Run lsusb and rtl_test -t | USB/SDR |
| SDR disappears from lsusb | Check dmesg, power-cycle, verify PoE/power budget | USB/power |
| USB error -110 / unable to enumerate | Full power-cycle; inspect 5 V supply and USB path | Power/USB hardware |
| Neighbor meters appear | Expected on watermeter/events; verify filter meter ID | RF/filter |
| Water total is wrong scale | Compare raw consumption against physical register | Meter scaling |

## Fast diagnostic sequence

# 1. Is the SDR present?
lsusb

# 2. Can rtl_433 decode R900 packets?
sudo systemctl stop watermeter
rtl_433 -f 915M -s 1024k -R 228 -F json
# Ctrl+C, then:
sudo systemctl start watermeter

# 3. Are raw packets reaching MQTT?
mosquitto_sub -h 127.0.0.1 -u watermeter -P '<MQTT_PASSWORD>' -t 'watermeter/events' -v

# 4. Is the filtered retained topic updating?
mosquitto_sub -h 127.0.0.1 -u watermeter -P '<MQTT_PASSWORD>' -t 'watermeter/home' -v

# 5. Are the services healthy?
systemctl status watermeter watermeter-filter mosquitto --no-pager

# 18. Security and maintenance

Use a dedicated MQTT account and keep anonymous access disabled.

Do not store the real MQTT password in shared documentation or source control.

If a password is exposed, rotate it with mosquitto_passwd and update rtl_433, the filter script, and Home Assistant.

Back up /etc/rtl_433, /etc/mosquitto, /usr/local/bin/watermeter-filter.sh, and both systemd service files.

Apply OS updates periodically and reboot when kernel/firmware updates require it.

# 19. Final verification checklist

☐ PoE splitter provides stable USB-C power and wired Ethernet to the Pi 5.

☐ vcgencmd get_throttled returns 0x0.

☐ lsusb shows the Nooelec/Realtek RTL-SDR.

☐ rtl_433 decodes Neptune-R900 packets with protocol 228.

☐ Meter ID 1584089202 is confirmed for this project.

☐ watermeter/events contains raw R900 events.

☐ watermeter/home contains only the project meter and is retained.

☐ Mosquitto, watermeter, and watermeter-filter services are enabled and active.

☐ Home Assistant connects to the Pi broker.

☐ Water Consumption shows the expected cumulative gallons.

☐ All MQTT entities are grouped under Neptune MACH 10 Water Meter.

☐ Water Consumption is configured as the Energy dashboard water source.

☐ Daily/weekly/monthly Utility Meter helpers are accumulating usage.